Alright, Ishaan — this is a great question because it forces you to think like a systems architect:
“Which enhancements are internal RTL refactors, and which ones ripple outward into verification, register maps, sequences, coverage, and scoreboards?”

Let’s categorize each feature cleanly across three buckets:

🟦 Category 1 — Changes that modify RTL but DO NOT impact verification architecture

These are internal structural changes that don’t alter the DUT interface or observable behavior from the outside.
Verification still sees the same ports, same timing model, same semantics.

✅ 1. Center-aligned PWM (up-down counter)

- Internal counter behavior changes

- Comparator logic unchanged

- Same ports, same duty semantics

- Verification only sees a different waveform shape, but no new signals or protocols

✅ 2. Multi-channel PWM (shared counter, multiple comparators)

- Still just “N outputs” and “N duty inputs”

- Verification scales linearly but architecture doesn’t change

- No new protocols or register interactions

✅ 3. Phase shifting (per-channel offset)

- Adds an adder before comparator

- No new handshake or protocol

- Verification only checks shifted waveforms

Why these don’t impact verification architecture

Because the interface contract stays the same.
Verification only needs to measure duty/phase, which it already does.

🟩 Category 2 — Changes that barely modify design (simple logic extensions)

These are “one more step in the pipeline” type changes.
They don’t change the architecture or interface, but they add small logic blocks.

🟩 1. Prescaler

- Just inserts a divider before the counter

- No new ports

- No new protocol

- Verification only sees a slower PWM frequency

🟩 2. Double-buffered duty register

- Adds shadow register + latch on rollover

- No new ports

- No new protocol

- Verification only needs to be aware that duty updates take effect at period boundaries

🟩 3. Synchronous start/stop

- Adds gating logic

- No new interface unless you expose sync_start

- Verification only checks alignment

Why these are “barely changes”

Because they don’t alter the structure of the design — they just refine timing or sequencing.

🟥 Category 3 — Changes that DO impact verification architecture

These introduce new interfaces, new protocols, new states, or new safety behavior.
Verification must add:

- new sequences

- new coverage points

- new monitors

- new scoreboard logic

- new register model fields

🟥 1. Dead-time generator (half-bridge outputs)

- Adds new outputs (pwm_high, pwm_low)

- Adds timing constraints

- Adds safety conditions

- Verification must check dead-time correctness, complementary behavior, and no shoot-through

🟥 2. Register interface (APB/AXI-Lite)

This is the biggest verification impact.

- Adds bus protocol

- Adds register model

- Adds sequences for reads/writes

- Adds coverage for register fields

- Adds scoreboard for register effects

- Adds UVM agent for the bus

🟥 3. Fault handling (fault override, fault latch)

- Adds new inputs (fault_in)

- Adds new states (faulted, cleared)

- Adds safety behavior

- Verification must test fault injection, latch behavior, recovery, and override priority

Why these impact verification

Because they fundamentally change the external contract of the DUT.

🧩 Summary Table

| Feature | Category | Reason |
| --- | --- | --- |
| Prescaler | 🟩 Barely changes design | Internal timing change only |
| Double-buffered duty | 🟩 Barely changes design | Internal sequencing only |
| Center-aligned PWM | 🟦 Design change, no verification impact | Same interface, different waveform |
| Multi-channel PWM | 🟦 Design change, no verification impact | Scales outputs, no new protocols |
| Phase shifting | 🟦 Design change, no verification impact | Adds offset logic only |
| Synchronous start/stop | 🟩 Barely changes design | Gating logic only |
| Dead-time generator | 🟥 Impacts verification | New outputs + safety timing |
| Register interface | 🟥 Impacts verification | New protocol + register model |
| Fault handling | 🟥 Impacts verification | New states + safety behavior |
| Programmable resolution | 🟦 Design change, no verification impact | Same interface, different max count |

🧠 Why this categorization matters

Because it tells you:

- which features you can safely add now without touching verification

- which ones require architectural planning

- which ones will force you to expand your UVM environment later

This is exactly how real IP teams plan feature roadmaps.

If you want, I can now help you:

- choose which features to implement first

- design the updated RTL architecture

- create a roadmap for when verification comes into play

Just tell me how you want to proceed.